1 ФОРМУВАННЯ VHDL ОПИСУ ФРАГМЕНТУ СХЕМИ

1.1 	Мета роботи

Вивчення структури VHDL опису комбінаційної схеми – базового компонента SoC. Отримання практичних навичок створення поведінкового опису комбінаційної схеми та її моделювання засобами мови VHDL.

1.2	Методичні вказівки з організації самостійної роботи студентів

Під час підготовки до лабораторної роботи слід ознайомитися з літературою [1-2], матеріалами конспекту лекцій та інформацією, наведеною в підрозділі 1.4.

1.3	Опис лабораторної установки

Cloud service for design of digital devices

1. Login Page:

https://www.edaplayground.com/login

2. The main window.

Open Wave Form after simulation

Options of debug and simulation

Tools and simulators

Libraries

Hardware description language (VHDL or Verilog)

Run simulation

Download file after simulation

Fig. 1. Main window

Examples of the projects

1.4 Порядок виконання роботи

Приклад опису фрагмента схеми пристрою (рис. __).

Listing of VHDL code for Testbench and project of the circuit are shown below.

| -- Testbench for Simple And-OR circuit design library IEEE; use IEEE.std_logic_1164.all; entity testbench is -- empty end testbench; architecture tb of testbench is -- DUT component component and_or_gate is port( a: in std_logic; b: in std_logic; c: in std_logic; q: out std_logic; q2: out std_logic); end component; signal a_in, b_in, c_in, q_out, q2_out: std_logic; begin -- Connect DUT DUT: and_or_gate port map(a_in, b_in, c_in, q_out, q2_out); process begin a_in <= '0'; b_in <= '0'; c_in <= '0'; wait for 1 ns; assert(q_out='1') report "Fail 0/0" severity error; a_in <= '0'; b_in <= '1'; c_in <= '1'; wait for 1 ns; assert(q_out='0') report "Fail 0/1" severity error; a_in <= '1'; b_in <= 'X'; c_in <= '1'; wait for 1 ns; assert(q_out='0') report "Fail 1/X" severity error; a_in <= '1'; b_in <= '1'; c_in <= '1'; wait for 1 ns; assert(q_out='0') report "Fail 1/1" severity error; -- Clear inputs a_in <= '0'; b_in <= '0'; c_in <= '0'; assert false report "Test done." severity note; wait; end process; end tb; | -- Simple And-OR circuit design library IEEE; use IEEE.std_logic_1164.all; entity and_or_gate is port( a: in std_logic; b: in std_logic; c: in std_logic; q: out std_logic; q2: out std_logic); end and_or_gate; architecture rtl of and_or_gate is begin process(a, b, c) is begin q <= a and b; q2 <= q or c; end process; end rtl; |
| --- | --- |

Рис. __ Результати моделювання

1.4 Порядок виконання роботи

1. Отримати у викладача індивідуальне завдання.

2. Здійснити опис функціональності.

3. Виконати моделювання проекту засобами мови VHDL.

4. Оформити звіт.

1.5 Варіанти завдання

1)

2)

3)

4)

5)

6)

7)

1.6	Контрольні запитання

1. Дайте визначення системи на кристалі.  Опишіть основні компоненти SoC.

2. Опишіть 3D-архітектури систем на кристалах.

3. Опишіть процес проектування систем на кристалах.

4. Опишіть криву графіка створення систем на кристалах, що включає фази проектування, налагодження і виробництва.

5. Опишіть технологію виготовлення і тестування цифрових систем в пакетах кристалів.

6. Опишіть нові тренди Gartner's Hype Emerging Technologies Cycle 2021.

7. Опишіть структуру мобільного спеціалізованого цифрового пристрою.

8. Опишіть архітектури кристалів програмованої логіки (PLD).

9. Опишіть архітектуру вентильних матриць FPGA.

10. Опишіть технології проектування цифрових систем на кристалах.

11. Опишіть особливості проектування спеціалізованих інтегральних схем ASIC.

2 ФОРМУВАННЯ VHDL ОПИСУ СУМАТОРА

2.1 	Мета роботи

Вивчення структури VHDL опису суматора – базового компонента SoC. Отримання практичних навичок створення поведінкового опису суматора та його моделювання засобами мови VHDL.

2.2	Методичні вказівки з організації самостійної роботи студентів

Під час підготовки до лабораторної роботи слід ознайомитися з літературою [1-4], матеріалами конспекту лекцій та інформацією, наведеною в підрозділі 2.4.

2.3	Опис лабораторної установки

Каскадний суматор (Ripple-carry adder) – це логічна схема, яка виконує додавання багатобітових двійкових чисел.

2.3.1 Приклад VHDL опису масштабованого каскадного суматора (Scalable Ripple Carry Adder) з використанням параметризації наведено нижче. В якості параметра використовується g_WIDTH – кількість розрядів або ширина вхідного слова.

library IEEE; use IEEE.std_logic_1164.all; entity full_adder is port (i_bit1, i_bit2, i_carry: in std_logic; -- Inputs o_sum, o_carry: out std_logic); -- Outputs end full_adder; architecture Equations of full_adder is signal A1, A2, A3 : std_logic; -- input signals begin -- Parallel assignment statements o_sum <= i_bit1 xor i_bit2 xor i_carry; A1 <= i_bit1 and i_bit2; -- gives g A2 <= i_bit1 and i_carry; A3 <= i_bit2 and i_carry; o_carry <= A1 or A2 or A3; end Equations; ------------------------------------------------------------------------------- -- File Downloaded from http://www.nandland.com ------------------------------------------------------------------------------- library ieee; use ieee.std_logic_1164.all; entity ripple_carry_adder is generic ( g_WIDTH : natural := 2 ); port ( i_add_term1 : in std_logic_vector(g_WIDTH-1 downto 0); i_add_term2 : in std_logic_vector(g_WIDTH-1 downto 0); -- o_result : out std_logic_vector(g_WIDTH downto 0) ); end ripple_carry_adder; architecture rtl of ripple_carry_adder is component full_adder is port ( i_bit1 : in std_logic; i_bit2 : in std_logic; i_carry : in std_logic; o_sum : out std_logic; o_carry : out std_logic); end component full_adder; signal w_CARRY : std_logic_vector(g_WIDTH downto 0); signal w_SUM : std_logic_vector(g_WIDTH-1 downto 0); begin w_CARRY(0) <= '0'; -- no carry input on first full adder SET_WIDTH : for ii in 0 to g_WIDTH-1 generate i_FULL_ADDER_INST : full_adder port map ( i_bit1 => i_add_term1(ii), i_bit2 => i_add_term2(ii), i_carry => w_CARRY(ii), o_sum => w_SUM(ii), o_carry => w_CARRY(ii+1) ); end generate SET_WIDTH; o_result <= w_CARRY(g_WIDTH) & w_SUM; -- VHDL Concatenation end rtl;

VHDL Testbench:

------------------------------------------------------------------------------- -- File Downloaded from http://www.nandland.com ------------------------------------------------------------------------------- library ieee; use ieee.std_logic_1164.all; entity ripple_carry_adder_tb is end ripple_carry_adder_tb; architecture rtl of ripple_carry_adder_tb is constant c_WIDTH : integer := 2; signal r_ADD_1 : std_logic_vector(c_WIDTH-1 downto 0) := (others => '0'); signal r_ADD_2 : std_logic_vector(c_WIDTH-1 downto 0) := (others => '0'); signal w_RESULT : std_logic_vector(c_WIDTH downto 0); component ripple_carry_adder is generic ( g_WIDTH : natural ); port ( i_add_term1 : in std_logic_vector(g_WIDTH-1 downto 0); i_add_term2 : in std_logic_vector(g_WIDTH-1 downto 0); -- o_result : out std_logic_vector(g_WIDTH downto 0) ); end component ripple_carry_adder; begin -- Instantiate the Unit Under Test (UUT) UUT : ripple_carry_adder generic map ( g_WIDTH => c_WIDTH ) port map ( i_add_term1 => r_ADD_1, i_add_term2 => r_ADD_2, o_result => w_RESULT ); -- Test bench is non-synthesizable process is begin r_ADD_1 <= "00"; r_ADD_2 <= "01"; wait for 10 ns; r_ADD_1 <= "10"; r_ADD_2 <= "01"; wait for 10 ns; r_ADD_1 <= "01"; r_ADD_2 <= "11"; wait for 10 ns; r_ADD_1 <= "11"; r_ADD_2 <= "11"; wait ; end process; end rtl;

2.3.2 Приклад VHDL опису масштабованого суматора з прискореним перенесенням (Carry Lookahead Adder) з використанням параметризації наведено нижче. В якості параметра використовується g_WIDTH – кількість розрядів або ширина вхідного слова.

Carry Lookahead Adder обчислює один або кілька бітів перенесення до виконання підсумовування, що зменшує час очікування для обчислення результату. Він призначений для паралельного формування бітів перенесення при додаванні двійкових чисел в суматор.

library IEEE; use IEEE.std_logic_1164.all; entity full_adder is port (i_bit1, i_bit2, i_carry: in std_logic; -- Inputs o_sum, o_carry: out std_logic); -- Outputs end full_adder; architecture Equations of full_adder is signal A1, A2, A3 : std_logic; -- input signals begin -- Parallel assignment statements o_sum <= i_bit1 xor i_bit2 xor i_carry; A1 <= i_bit1 and i_bit2; -- gives g A2 <= i_bit1 and i_carry; A3 <= i_bit2 and i_carry; o_carry <= A1 or A2 or A3; end Equations; ------------------------------------------------------------------------------- -- File Downloaded from http://www.nandland.com ------------------------------------------------------------------------------- library ieee; use ieee.std_logic_1164.all; entity carry_lookahead_adder is generic ( g_WIDTH : natural -- все неотрицательные числа ); port ( i_add1 : in std_logic_vector(g_WIDTH-1 downto 0); i_add2 : in std_logic_vector(g_WIDTH-1 downto 0); -- o_result : out std_logic_vector(g_WIDTH downto 0) ); end carry_lookahead_adder; architecture rtl of carry_lookahead_adder is component full_adder is port ( i_bit1 : in std_logic; i_bit2 : in std_logic; i_carry : in std_logic; o_sum : out std_logic; o_carry : out std_logic); end component full_adder; signal w_G : std_logic_vector(g_WIDTH-1 downto 0); -- Generate signal w_P : std_logic_vector(g_WIDTH-1 downto 0); -- Propagate signal w_C : std_logic_vector(g_WIDTH downto 0); -- Carry signal w_SUM : std_logic_vector(g_WIDTH-1 downto 0); begin -- Create the Full Adders GEN_FULL_ADDERS : for ii in 0 to g_WIDTH-1 generate FULL_ADDER_INST : full_adder port map ( i_bit1 => i_add1(ii), i_bit2 => i_add2(ii), i_carry => w_C(ii), o_sum => w_SUM(ii), o_carry => open ); end generate GEN_FULL_ADDERS; -- Create the Generate (G) Terms: Gi=Ai*Bi -- Create the Propagate Terms: Pi=Ai+Bi -- Create the Carry Terms: GEN_CLA : for jj in 0 to g_WIDTH-1 generate w_G(jj) <= i_add1(jj) and i_add2(jj); w_P(jj) <= i_add1(jj) or i_add2(jj); w_C(jj+1) <= w_G(jj) or (w_P(jj) and w_C(jj)); end generate GEN_CLA; w_C(0) <= '0'; -- no carry input o_result <= w_C(g_WIDTH) & w_SUM; -- VHDL Concatenation end rtl;

VHDL Testbench:

------------------------------------------------------------------------------- -- File Downloaded from http://www.nandland.com ------------------------------------------------------------------------------- library ieee; use ieee.std_logic_1164.all; entity carry_lookahead_adder_tb is end carry_lookahead_adder_tb; architecture behave of carry_lookahead_adder_tb is constant c_WIDTH : integer := 3; signal r_ADD_1 : std_logic_vector(c_WIDTH-1 downto 0) := (others => '0'); signal r_ADD_2 : std_logic_vector(c_WIDTH-1 downto 0) := (others => '0'); signal w_RESULT : std_logic_vector(c_WIDTH downto 0); component carry_lookahead_adder is generic ( g_WIDTH : natural ); port ( i_add1 : in std_logic_vector(g_WIDTH-1 downto 0); i_add2 : in std_logic_vector(g_WIDTH-1 downto 0); o_result : out std_logic_vector(g_WIDTH downto 0) ); end component carry_lookahead_adder; begin -- Instantiate the Unit Under Test (UUT) UUT : carry_lookahead_adder generic map ( g_WIDTH => c_WIDTH ) port map ( i_add1 => r_ADD_1, i_add2 => r_ADD_2, o_result => w_RESULT ); -- Test bench is non-synthesizable process is begin r_ADD_1 <= "000"; r_ADD_2 <= "001"; wait for 10 ns; r_ADD_1 <= "100"; r_ADD_2 <= "010"; wait for 10 ns; r_ADD_1 <= "010"; r_ADD_2 <= "110"; wait for 10 ns; r_ADD_1 <= "111"; r_ADD_2 <= "111"; wait ; end process; end behave;

2.4 Порядок виконання роботи

1. Отримати у викладача індивідуальне завдання.

2. Здійснити опис функціональності.

3. Виконати моделювання проекту засобами мови VHDL.

4. Оформити звіт.

2.5 Варіанти завдання

1) п. 2.3.1, значення параметра g_WIDTH : natural := 4.

2) п. 2.3.1, значення параметра g_WIDTH : natural := 8.

3) п. 2.3.1, значення параметра g_WIDTH : natural := 16.

4) п. 2.3.2, значення параметра g_WIDTH : natural := 4.

5) п. 2.3.2, значення параметра g_WIDTH : natural := 8.

6) п. 2.3.2, значення параметра g_WIDTH : natural := 16.

2.6	Контрольні запитання

1. Опишіть алгоритм роботи каскадного суматора.

2. Опишіть алгоритм роботи суматора з прискореним перенесенням.

3. Опишіть алгоритм роботи напівсуматора.

4. Опишіть алгоритм роботи мультиплексора та варіанти опису поведінки мультиплексора мовою VHDL.

5. Опишіть алгоритм роботи демультиплексора та варіанти опису поведінки демультиплексора мовою VHDL.

3 ФОРМУВАННЯ VHDL ОПИСУ ФРАГМЕНТУ ПОСЛІДОВНОСТНОЇ СХЕМИ

3.1 	Мета роботи

Вивчення структури VHDL опису послідовностної схеми – базового базового компонента SoC. Отримання практичних навичок створення поведінкового опису послідовностної схеми та її моделювання засобами мови VHDL.

3.2	Методичні вказівки з організації самостійної роботи студентів

Під час підготовки до лабораторної роботи слід ознайомитися з літературою [1-4], матеріалами конспекту лекцій та інформацією, наведеною в підрозділі 2.4.

3.3	Опис лабораторної установки

3.3.1 Регістр зсуву - це регістр, який використовується для зсуву збереженого біта в одному або обох напрямках. У наведеному нижче прикладі реалізовано регістр зсуву, який можна використовувати для зсуву даних в обох напрямках. Далі він може використовуватися як паралельно-послідовний або послідовно-паралельний перетворювач.

| -- Testbench automatically generated online -- at https://vhdl.lapinoo.net -- Generation date : 27.9.2020 21:52:10 UTC library ieee; use ieee.std_logic_1164.all; entity tb_shift_register is end tb_shift_register; architecture tb of tb_shift_register is component shift_register generic ( n : natural ); port (clk : in std_logic; reset : in std_logic; ctrl : in std_logic_vector (1 downto 0); data : in std_logic_vector (n-1 downto 0); q_reg : out std_logic_vector (n-1 downto 0)); end component; constant n : integer := 8; signal clk : std_logic; signal reset : std_logic; signal ctrl : std_logic_vector (1 downto 0); signal data : std_logic_vector (n-1 downto 0); signal q_reg : std_logic_vector (n-1 downto 0); constant TbPeriod : time := 100 ns; -- EDIT Put right period here signal TbClock : std_logic := '0'; signal TbSimEnded : std_logic := '0'; begin dut : shift_register generic map ( n => n ) port map (clk => clk, reset => reset, ctrl => ctrl, data => data, q_reg => q_reg); -- Clock generation TbClock <= not TbClock after TbPeriod/2 when TbSimEnded /= '1' else '0'; -- EDIT: Check that clk is really your main clock signal clk <= TbClock; stimuli : process begin -- EDIT Adapt initialization as needed ctrl <= (others => '0'); data <= (others => '0'); -- Reset generation -- EDIT: Check that reset is really your reset signal reset <= '1'; wait for 10 ns; reset <= '0'; wait for 10 ns; -- EDIT Add stimuli here ctrl <="00"; data <= "11010101"; wait for 10 * TbPeriod; ctrl <="01"; data <= "10010101"; wait for 10 * TbPeriod; ctrl <="10"; data <= "10001111"; wait for 10 * TbPeriod; ctrl <="11"; data <= "10101111"; wait for 10 * TbPeriod; -- Stop the clock and hence terminate the simulation TbSimEnded <= '1'; wait; end process; end tb; | -- shift_register.vhd -- created by : Meher Krishna Patel -- date : 22-Dec-16 -- Functionality: -- load data and shift it data to left and right -- parallel to serial conversion (i.e. first load, then shift) -- serial to parallel conversion (i.e. first shift, then read) -- inputs: -- ctrl : to load-data and shift operations (right and left shift) -- data : it is the data to be shifted -- q_reg : store the outputs library ieee; use ieee.std_logic_1164.all; entity shift_register is generic (N :integer :=8); port( clk, reset : in std_logic; ctrl : in std_logic_vector(1 downto 0); data : in std_logic_vector(N-1 downto 0); q_reg : out std_logic_vector(N-1 downto 0) ); end shift_register; architecture arch of shift_register is signal s_reg, s_next : std_logic_vector(N-1 downto 0); begin process(clk, reset) begin if(reset='1') then s_reg <= (others=>'0'); -- clear the content elsif (clk'event and clk='1') then s_reg <= s_next; -- otherwise save the next state end if; end process; process (ctrl, s_reg) begin case ctrl is when "00" => s_next <= s_reg; -- no operation (to read data for serial to parallel) when "01" => s_next <= data(N-1) & s_reg(N-1 downto 1); -- right shift when "10" => s_next <= s_reg(N-2 downto 0) & data(0); -- left shift when others => s_next <= data; -- load data (for parallel to serial) end case; end process; q_reg <= s_reg; end arch; |
| --- | --- |

3.3.2 Регістр зсуву з лінійним зворотним зв’язком (LFSR) можна використовувати як "генератор псевдовипадкових чисел", які генеруються на основі початкових значень для LFSR. Поліном зворотного зв'язку реалізується відповідно до значення N (табл.__). Для N=3 поліном зворотного зв'язку реалізований як x^3 + x^2 + 1. Щоб змінити поліном зворотного зв'язку, необхідно використовувати нову кількість бітів (N) та змінити значення feedback_value відповідно до нового значення «N».

| -- Testbench automatically generated online -- at https://vhdl.lapinoo.net -- Generation date : 21.9.2020 21:50:40 UTC library ieee; use ieee.std_logic_1164.all; entity tb_rand_num_generator is end tb_rand_num_generator; architecture tb of tb_rand_num_generator is component rand_num_generator port (clk : in std_logic; reset : in std_logic; q : out std_logic_vector (3 downto 0)); end component; signal clk : std_logic; signal reset : std_logic; signal q : std_logic_vector (3 downto 0); constant TbPeriod : time := 1000 ns; -- EDIT Put right period here signal TbClock : std_logic := '0'; signal TbSimEnded : std_logic := '0'; begin dut : rand_num_generator port map (clk => clk, reset => reset, q => q); -- Clock generation TbClock <= not TbClock after TbPeriod/2 when TbSimEnded /= '1' else '0'; -- EDIT: Check that clk is really your main clock signal clk <= TbClock; stimuli : process begin -- EDIT Adapt initialization as needed -- Reset generation -- EDIT: Check that reset is really your reset signal reset <= '1'; wait for 100 ns; reset <= '0'; wait for 100 ns; -- EDIT Add stimuli here wait for 100 * TbPeriod; -- Stop the clock and hence terminate the simulation TbSimEnded <= '1'; wait; end process; end tb; -- Configuration block below is required by some simulators. Usually no need to edit. configuration cfg_tb_rand_num_generator of tb_rand_num_generator is for tb end for; end cfg_tb_rand_num_generator; | -- rand_num_generator.vhd -- created by : Meher Krishna Patel -- date : 22-Dec-16 -- Feedback polynomial : x^3 + x^2 + 1 -- maximum length : 2^3 - 1 = 7 -- if generic value is changed, -- then choose the correct Feedback polynomial i.e. change 'feedback_value' pattern library ieee; use ieee.std_logic_1164.all; entity rand_num_generator is generic (N :integer := 3); port( clk, reset : in std_logic; q : out std_logic_vector(N downto 0) -- output of LFSR i.e. random number ); end rand_num_generator; architecture arch of rand_num_generator is signal r_reg, r_next : std_logic_vector(N downto 0); signal feedback_value : std_logic; -- based on feedback polynomial begin process(clk, reset) begin if(reset='1') then -- set initial value to '1'. r_reg(0) <= '1'; -- 0th bit = 1 r_reg(N downto 1) <= (others=>'0'); -- other bits are 0 elsif (clk'event and clk='1') then r_reg <= r_next; -- otherwise save the next state end if; end process; -- N = 3 -- Feedback polynomial : x^3 + x^2 + 1 -- total sequences (maximum) : 2^3 - 1 = 7 feedback_value <= r_reg(3) xor r_reg(2) xor r_reg(0); -- N = 4 -- feedback_value <= r_reg(4) xor r_reg(3) xor r_reg(0); -- N = 5, maximum length = 28 (not 31) -- feedback_value <= r_reg(5) xor r_reg(3) xor r_reg(0); -- N = 9 -- feedback_value <= r_reg(9) xor r_reg(5) xor r_reg(0); r_next <= feedback_value & r_reg(N downto 1); q <= r_reg; end arch; |
| --- | --- |

3.4 Порядок виконання роботи

1. Отримати у викладача індивідуальне завдання.

2. Здійснити опис функціональності.

3. Виконати моделювання проекту засобами мови VHDL.

4. Оформити звіт.

3.5 Варіанти завдання

1) п. 3.3.1, значення параметра N :integer :=4.

2) п. 3.3.1, значення параметра N :integer :=8.

3) п. 3.3.1, значення параметра N :integer :=16.

4) п. 3.3.2, значення параметра N :integer :=3.

2) п. 3.3.2, значення параметра N :integer :=4.

6) п. 3.3.2, значення параметра N :integer :=5.

7) п. 3.3.2, значення параметра N :integer :=9.

3.6	Контрольні запитання

1. Опишіть алгоритм роботи регістра зсуву.

2. Опишіть алгоритм роботи регістра зсуву з лінійним зворотним зв’язком (LFSR).

3. Опишіть поведінкові моделі системного рівня проектування.

4. Опишіть структурні моделі системного рівня проектування.

5. Опишіть поведінкові моделі процесорного рівня проектування.

6. Опишіть структурні моделі процесорного рівня проектування.

7. Опишіть процедуру синтезу на процесорному рівні.

8. Опишіть завдання, які вирішуються при перетворенні поведінкової моделі в оптимізовану системну платформу.

4 ФОРМУВАННЯ VHDL ОПИСУ МОДУЛЯ ПАМ’ЯТІ

4.1 	Мета роботи

Вивчення структури VHDL опису модуля пам’яті – базового компонента SoC. Отримання практичних навичок створення поведінкового опису модуля пам’яті та її моделювання засобами мови VHDL.

4.2	Методичні вказівки з організації самостійної роботи студентів

Під час підготовки до лабораторної роботи слід ознайомитися з літературою [1-4], матеріалами конспекту лекцій та інформацією, наведеною в підрозділі 2.4.

4.3	Опис лабораторної установки

4.3.1 ПЗУ – це пристрої, які використовуються для постійного зберігання інформації. У наступному прикладі ПЗУ реалізовано на FPGA для зберігання шаблону відображення для семисегментного пристрою.

| -- Testbench automatically generated online -- at https://vhdl.lapinoo.net -- Generation date : 1.10.2020 20:29:40 UTC library ieee; use ieee.std_logic_1164.all; entity tb_ROM_sevenSegment is end tb_ROM_sevenSegment; architecture tb of tb_ROM_sevenSegment is component ROM_sevenSegment generic( addr_bits : integer := 4; data_width : natural := 7 ); port (addr : in std_logic_vector (addr_bits-1 downto 0); data : out std_logic_vector (data_width-1 downto 0)); end component; constant addr_bits : integer := 4; constant data_width : integer := 7; signal addr : std_logic_vector (addr_bits-1 downto 0); signal data : std_logic_vector (data_width-1 downto 0); constant TbPeriod : time := 100 ns; -- EDIT Put right period here signal TbClock : std_logic := '0'; signal TbSimEnded : std_logic := '0'; begin dut : ROM_sevenSegment generic map ( addr_bits => addr_bits, -- required bits to store 16 elements data_width => data_width -- each element has 7-bits ) port map (addr => addr, data => data); -- Clock generation TbClock <= not TbClock after TbPeriod/2 when TbSimEnded /= '1' else '0'; stimuli : process begin -- EDIT Adapt initialization as needed addr <= (others => '0'); -- EDIT Add stimuli here addr <= "1111"; wait for 10 * TbPeriod; addr <= "1011"; wait for 10 * TbPeriod; -- Stop the clock and hence terminate the simulation TbSimEnded <= '1'; wait; end process; end tb; -- Configuration block below is required by some simulators. Usually no need to edit. configuration cfg_tb_ROM_sevenSegment of tb_ROM_sevenSegment is for tb end for; end cfg_tb_ROM_sevenSegment; | --ROM_sevenSegment.vhd -- created by : Meher Krishna Patel -- date : 25-Dec-16 -- Functionality: -- seven-segment display format for Hexadecimal values (i.e. 0-F) are stored in ROM -- ports: -- addr : input port for getting address -- data : ouput data at location 'addr' -- addr_width : total number of elements to store (put exact number) -- addr_bits : bits requires to store elements specified by addr_width -- data_width : number of bits in each elements library ieee; use ieee.std_logic_1164.all; use ieee.numeric_std.all; entity ROM_sevenSegment is generic( addr_width : integer := 16; -- store 16 elements addr_bits : integer := 4; -- required bits to store 16 elements data_width : integer := 7 -- each element has 7-bits ); port( addr : in std_logic_vector(addr_bits-1 downto 0); data : out std_logic_vector(data_width-1 downto 0) ); end ROM_sevenSegment; architecture arch of ROM_sevenSegment is type rom_type is array (0 to addr_width-1) of std_logic_vector(data_width-1 downto 0); -- ROM type signal sevenSegment_ROM : rom_type := ( "1000000", -- 0, active low i.e. 0:display & 1:no display "1111001", -- 1 "0100100", -- 2 "0110000", -- 3 "0011001", -- 4 "0010010", -- 5 "0000010", -- 6 "1111000", -- 7 "0000000", -- 8 "0010000", -- 9 "0001000", -- a "0000011", -- b "1000110", -- c "0100001", -- d "0000110", -- e "0001110" -- f ); begin data <= sevenSegment_ROM(to_integer(unsigned(addr))); end arch; |
| --- | --- |

4.3.2 Однопортова оперативна пам’ять має один вхідний порт (рядок адреси), який використовується як для зберігання, так і для отримання даних.

| -- Testbench automatically generated online -- at https://vhdl.lapinoo.net -- Generation date : 4.10.2020 21:22:48 UTC library ieee; use ieee.std_logic_1164.all; entity tb_single_port_RAM is end tb_single_port_RAM; architecture tb of tb_single_port_RAM is component single_port_RAM generic( addr_width : integer := 2; data_width : integer := 3 ); port (clk : in std_logic; we : in std_logic; addr : in std_logic_vector (addr_width-1 downto 0); din : in std_logic_vector (data_width-1 downto 0); dout : out std_logic_vector (data_width-1 downto 0)); end component; constant addr_width : integer := 2; constant data_width : integer := 3; signal clk : std_logic; signal we : std_logic; signal addr : std_logic_vector (addr_width-1 downto 0); signal din : std_logic_vector (data_width-1 downto 0); signal dout : std_logic_vector (data_width-1 downto 0); constant TbPeriod : time := 1000 ns; -- EDIT Put right period here signal TbClock : std_logic := '0'; signal TbSimEnded : std_logic := '0'; begin dut : single_port_RAM generic map ( addr_width => addr_width, data_width => data_width ) port map (clk => clk, we => we, addr => addr, din => din, dout => dout); -- Clock generation TbClock <= not TbClock after TbPeriod/2 when TbSimEnded /= '1' else '0'; -- EDIT: Check that clk is really your main clock signal clk <= TbClock; stimuli : process begin -- EDIT Adapt initialization as needed we <= '0'; addr <= (others => '0'); din <= (others => '0'); -- EDIT Add stimuli here we <= '1'; addr <= "01"; din <= "111"; wait for 10 * TbPeriod; we <= '0'; addr <= "01"; wait for 10 * TbPeriod; -- Stop the clock and hence terminate the simulation TbSimEnded <= '1'; wait; end process; end tb; | -- single_port_RAM.vhd -- created by : Meher Krishna Patel -- date : 26-Dec-16 -- Functionality: -- store and retrieve data from single port RAM -- ports: -- we : write enable -- addr : input port for getting address -- din : input data to be stored in RAM -- data : output data read from RAM -- addr_width : total number of elements to store (put exact number) -- addr_bits : bits requires to store elements specified by addr_width -- data_width : number of bits in each elements library ieee; use ieee.std_logic_1164.all; use ieee.numeric_std.all; entity single_port_RAM is generic ( addr_width : integer := 2; data_width : integer := 3 ); port( clk: in std_logic; we : in std_logic; addr : in std_logic_vector(addr_width-1 downto 0); din : in std_logic_vector(data_width-1 downto 0); dout : out std_logic_vector(data_width-1 downto 0) ); end single_port_RAM; architecture arch of single_port_RAM is type ram_type is array (2**addr_width-1 downto 0) of std_logic_vector (data_width-1 downto 0); signal ram_single_port : ram_type; begin process(clk) begin if (clk'event and clk='1') then if (we='1') then -- write data to address 'addr' --convert 'addr' type to integer from std_logic_vector ram_single_port(to_integer(unsigned(addr))) <= din; end if; end if; end process; -- read data from address 'addr' -- convert 'addr' type to integer from std_logic_vector dout<=ram_single_port(to_integer(unsigned(addr))); end arch; |
| --- | --- |

4.3.3 Двопортова оперативна пам'ять використовує два вхідні порти для операцій читання та запису.

| -- Testbench automatically generated online -- at https://vhdl.lapinoo.net -- Generation date : 4.10.2020 21:41:20 UTC library ieee; use ieee.std_logic_1164.all; entity tb_dual_port_RAM is end tb_dual_port_RAM; architecture tb of tb_dual_port_RAM is component dual_port_RAM generic( addr_width : integer := 2; data_width : integer := 3 ); port (clk : in std_logic; we : in std_logic; addr_wr : in std_logic_vector (addr_width-1 downto 0); addr_rd : in std_logic_vector (addr_width-1 downto 0); din : in std_logic_vector (data_width-1 downto 0); dout : out std_logic_vector (data_width-1 downto 0)); end component; constant addr_width : integer := 2; constant data_width : integer := 3; signal clk : std_logic; signal we : std_logic; signal addr_wr : std_logic_vector (addr_width-1 downto 0); signal addr_rd : std_logic_vector (addr_width-1 downto 0); signal din : std_logic_vector (data_width-1 downto 0); signal dout : std_logic_vector (data_width-1 downto 0); constant TbPeriod : time := 1000 ns; -- EDIT Put right period here signal TbClock : std_logic := '0'; signal TbSimEnded : std_logic := '0'; begin dut : dual_port_RAM generic map ( addr_width => addr_width, data_width => data_width ) port map (clk => clk, we => we, addr_wr => addr_wr, addr_rd => addr_rd, din => din, dout => dout); -- Clock generation TbClock <= not TbClock after TbPeriod/2 when TbSimEnded /= '1' else '0'; -- EDIT: Check that clk is really your main clock signal clk <= TbClock; stimuli : process begin -- EDIT Adapt initialization as needed we <= '0'; addr_wr <= (others => '0'); addr_rd <= (others => '0'); din <= (others => '0'); -- EDIT Add stimuli here we <= '1'; addr_wr <= "01"; din <= "011"; wait for 10 * TbPeriod; we <= '1'; addr_wr <= "10"; din <= "111"; wait for 10 * TbPeriod; we <= '0'; addr_rd <= "01"; wait for 10 * TbPeriod; we <= '0'; addr_rd <= "10"; wait for 10 * TbPeriod; -- Stop the clock and hence terminate the simulation TbSimEnded <= '1'; wait; end process; end tb; | -- dual_port_RAM.vhd -- created by : Meher Krishna Patel -- date : 26-Dec-16 -- Functionality: -- store and retrieve data from single port RAM -- ports: -- we : write enable -- addr_wr : address for writing data -- addr_rd : address for reading -- din : input data to be stored in RAM -- data : output data read from RAM -- addr_width : total number of elements to store (put exact number) -- addr_bits : bits requires to store elements specified by addr_width -- data_width : number of bits in each elements library ieee; use ieee.std_logic_1164.all; use ieee.numeric_std.all; entity dual_port_RAM is generic ( addr_width : integer := 2; data_width : integer := 3 ); port( clk: in std_logic; we : in std_logic; addr_wr, addr_rd : in std_logic_vector(addr_width-1 downto 0); din : in std_logic_vector(data_width-1 downto 0); dout : out std_logic_vector(data_width-1 downto 0) ); end dual_port_RAM; architecture arch of dual_port_RAM is type ram_type is array (2**addr_width-1 downto 0) of std_logic_vector (data_width-1 downto 0); signal ram_dual_port : ram_type; begin process(clk) begin if (clk'event and clk='1') then if (we='1') then -- write data to address 'addr_wr' -- convert 'addr_wr' type to integer from std_logic_vector ram_dual_port(to_integer(unsigned(addr_wr))) <= din; end if; end if; end process; -- get address for reading data from 'addr_rd' -- convert 'addr_rd' type to integer from std_logic_vector dout<=ram_dual_port(to_integer(unsigned(addr_rd))); end arch; |
| --- | --- |

4.4 Порядок виконання роботи

1. Отримати у викладача індивідуальне завдання.

2. Здійснити опис функціональності.

3. Виконати моделювання проекту засобами мови VHDL.

4. Оформити звіт.

4.5 Варіанти завдання

1) п. 4.3.1.

2) п. 4.3.2.

3) п. 4.3.3.

4.6	Контрольні запитання

- Опишіть алгоритм роботи модуля пам’яті.

- Поясніть архітектуру і опис регістрових файлів мовою VHDL.

- Поясніть опис семисегментного дисплею мовою VHDL.

- Поясніть опис ROM (ПЗУ) мовою VHDL.

- Поясніть опис однопортової RAM мовою VHDL.

- Поясніть опис двопортової RAM мовою VHDL.

- Поясніть опис модуля FIFO мовою VHDL.